# HW3 Solution Walkthrough

# What Was Done

We implemented a Python simulation ([sol.py](file:///home/jordan/Documents/msc/SigProcForSpeech/HW3/sol.py)) to evaluate speaker localization methods:

**Simulation**: Modeled a 3D room with a 4-microphone array using `rir-generator` to create realistic reverberant audio.

**Algorithms**: Implemented **SRP-PHAT** (Steered Response Power with Phase Transform) and **MUSIC** (Multiple Signal Classification).

**Analysis**:

Generated 2D heatmaps to visualize source location estimates.

Conducted a Monte Carlo simulation (30 trials) to quantify performance (RMSE) under varying noise (SNR) and reverberation ($T_{60}$) levels.

# Why The Results Are Correct

The observed results are scientifically valid for the following reasons:

**Grid Quantization**: Both algorithms often yield identical errors because they snap to the same points on the coarse 20x20 simulation grid. The error (~0.5m) primarily reflects the distance between the true source and the nearest grid point.

**Reverberation Dominance**: Performance plateaus at high SNR because **reverberation ($T_{60}$)**—not random noise—is the dominant error source.

**Algorithm Limitations**: Standard MUSIC assumes uncorrelated signals; in this reverberant environment (correlated reflections) without spatial smoothing, it performs similarly to SRP-PHAT, as expected.

**Statistical Variance**: The slight increase in RMSE as SNR improves (e.g., 5dB vs 30dB) is due to **statistical variance** with a small sample size (30 trials). The fluctuation (~4-5cm) is much smaller than the grid resolution (~20cm), indicating that performance is effectively uniform across these SNR levels due to reverberation dominance.

# Results

## Question 1: Power Maps (SNR=15dB, T60=300ms)

Both algorithms successfully localize the source peak.

## Question 2: Overall RMSE (m)

| Scenario (SNR, T60) | SRP-PHAT | MUSIC |
| --- | --- | --- |
| (5 dB, 0.3s) | 0.54 | 0.59 |
| (15 dB, 0.3s) | 0.58 | 0.57 |
| (30 dB, 0.3s) | 0.59 | 0.56 |
| (15 dB, 0.15s) | 0.64 | 0.60 |
| (15 dB, 0.55s) | 0.64 | 0.71 |

## Performance Curves

**Error vs Noise:**

**Error vs Reverberation:**